Project description:

Pathfinding is fundamental to robotics and autonomous navigation, requiring

rapid computation of optimal paths. While A* is the gold standard algorithm,

software implementations face latency challenges in time-critical applications.

This paper presents an FPGA-based solution achieving significant performance

improvements.

Our implementation extends traditional 4-connected A* to support 8-connected

motion, allowing diagonal movements that reduce path length by 30-50% while

maintaining optimality. We employ the Octile distance heuristic

[h(x,y) = max(dx,dy)×10 + min(dx,dy)×4], proven admissible for 8-connected

grids, ensuring optimal paths.

The hardware architecture features a ten-state finite state machine managing

initialization, priority-based node selection, neighbor expansion, and path

reconstruction. Priority queue operations use linear search optimized for

16×16 grids, trading algorithmic complexity for hardware simplicity and

predictable timing. Node data structures are stored in on-chip BRAM,

eliminating external memory latency.

Resource utilization on Xilinx 7-series FPGAs requires 2,847 LUTs, 1,923

flip-flops, and 3 BRAM blocks, achieving 178 MHz maximum frequency.

Comparative evaluation against Python implementation across seven diverse

test cases validates correctness through exact matching of nodes expanded

and path costs. Performance measurements demonstrate 13-36× speedup, with

hardware execution times of 3.45-85.2 microseconds versus 125-1150

microseconds in software.

Comprehensive testing includes simple diagonal paths, spiral mazes, random

obstacles, snake patterns, room layouts, diagonal corridors, and no-path

scenarios. All cases execute with deterministic timing and proven correctness,

validating both functional design and performance benefits for real-time

pathfinding applications in autonomous systems.

Verilog Screenshots